# Servo PWM Control using Op-Amp and 555 Timer

## Project Overview

This project demonstrates the control of a servo motor using Pulse Width Modulation (PWM) generated through a 555 Timer IC and controlled with the help of an Operational Amplifier (Op-Amp) comparator circuit.

The system allows precise servo angle control by varying the PWM pulse width using a potentiometer input. The Op-Amp processes the input signal and influences the PWM generation of the 555 timer, which drives the servo motor accordingly.

## Objectives

- To understand the working of Pulse Width Modulation (PWM)
- To generate PWM signals using a 555 Timer IC
- To control servo motor position using PWM
- To use an Op-Amp comparator for signal conditioning
- To simulate the circuit using Tinkercad
- To study practical applications of PWM-based motor control systems

## Introduction

Servo motors are widely used in automation, robotics, industrial control systems, and embedded applications where accurate angular position control is required.

Pulse Width Modulation (PWM) is one of the most common techniques used for controlling servo motors. In this project, a 555 timer is configured to generate PWM signals, while an Op-Amp comparator is used to process the input control voltage.

By adjusting the potentiometer, the pulse width of the PWM signal changes, resulting in different servo motor angles.

## What is PWM?

Pulse Width Modulation (PWM) is a technique used to vary the effective power delivered to electronic devices by changing the width of pulses while maintaining a constant frequency.

• 1 ms pulse width corresponds to approximately 0°
• 1.5 ms pulse width corresponds to approximately 90°
• 2 ms pulse width corresponds to approximately 180°

The servo motor continuously monitors the PWM signal and rotates its shaft according to the pulse width.

## Working Principle

1. The potentiometer provides a variable analog input voltage.
2. The Op-Amp acts as a comparator and conditions the input signal.
3. The conditioned signal influences the PWM generation circuit.
4. The 555 Timer operates in astable mode to generate PWM pulses.
5. The pulse width changes according to the potentiometer position.
6. The servo motor receives the PWM signal and rotates to the corresponding angle.

## Applications

- Robotics
- Industrial Automation
- Embedded Systems
- RC Vehicles
- Servo Positioning Systems
- Mechatronics Projects
- Motor Control Applications

## Advantages

- Simple and low-cost implementation
- Easy to understand and simulate
- Accurate servo angle control
- Useful for learning PWM concepts
- Widely applicable in automation systems

## Future Improvements

- Microcontroller-based PWM generation
- Digital angle display
- IoT-based remote control
- Closed-loop feedback system
- Mobile app control
- Wireless servo operation

## Conclusion

This project successfully demonstrates servo motor angle control using PWM generated by a 555 Timer IC along with an Op-Amp comparator circuit.

The project helps in understanding the practical implementation of PWM techniques and servo motor interfacing, which are essential concepts in embedded systems, automation, and robotics.

## Author

Velmurugan K

LinkedIn:
www.linkedin.com/in/velmurugan-k-69b11128b

## License

This project is licensed under the MIT License.